- PCCOM Professional

------ OCX & DLL ------

****************************************NOTE****************************************

- The methods / API’s defined here are the same in using the PCCOM Professional OCX & DLL.

**************************************************************************************

- PCCOM Professional is broken to 3 Major Categories:

- Initialization Methods / Functions

- Sending Data Methods / Functions

- Receiving  Data Methods / Functions

- Miscellaneous Functions

- Event Mapping

- Initialization Methods or Functions:

- 1.  {Long }commopen ( {Short}Portno);

-this method / API should be always called first before performing any other functions. This will make the specified port open for any communication functions. Use the “commclose” method / API to close the opened portno.

Result Values:

0    -OK

-1    -Invalid COM Port or COM Port is unavailable/busy

Ex.  ret=ID1.commopen ( 2 ); //OCX version

ret=commopen( 2); //DLL version

- 2. {Long }comminit ( {Short}Portno, {Long}Baudrate, {Short}Databits, {String}Parity, {Short}Stopbits,);

-this method / API is used to initialize the opened COM port to the specified settings. The possible values are:

For Portno: 1-xx

For BaudRate: 110, 300, 600, 1200, 2400….etc

For Databits: 5, 6, 7, and 8

For Parity: EVEN (even parity), MARK (mark parity), NO (no parity), ODD (odd parity), SPACE (space

parity)

For Stopbits: 1 (one stop bit), 2 (two stop bits) and 3 (1.5 stop bits)

Ex.  ret=ID1.comminit (2, 9600, 8, “NO”, 1);  //OCX version

ret= comminit (2, 9600, 8, “NO”, 1);  //DLL version

- 3. {Long }commclose ( {Short}Portno);

-this method / API is used  to close a opened COM port, previously opened by the “commopen” method / API.

Ex.  ret=ID1.commclose ( 2); //OCX version

ret= commclose ( 2); //DLL version

- Sending Data Methods or Functions:

***************************************NOTE*****************************************

- In order to use this methods properly you must first open the COM port and initialize it using “commopen” and “comminit”.

*************************************************************************************

- 1. {Long }commtxblk ( {Short}Portno, {String}Buffer, {Long}Length );

-this method / API is used for  sending a block of data using the specified COM port. The length or number of bytes to send must also be specified.

Ex.  ret=ID1.commtxblk(2, buf, 20); //OCX version

ret=commtxblk(2, buf, 20); //DLL version

- 2. {Long }commKermitTxFile ( {Short}Portno,  {String}Filename with path);

-this method / API is used for sending the specified file using the specified COM port. The file is sent via the Kermit Protocol.

Result Values:

- –OK

-1      -Invalid COM Port or COM Port is unavailable/busy

-2     -Bad Packet

-3     -Bad Sequence

-4     -Error on Transmission

-5     -Packet end error

-7     -File not Found

Ex.  ret=ID1.commKermitTxFile (2 , ”c:\autoexec.bat”); //OCX version

ret= commKermitTxFile (2 , ”c:\autoexec.bat”); //DLL version

- 3. {Long }commZmodemTxFile ( {Short}Portno,  {String}Filename with path);

-this method / API is used for sending the specified file using the specified COM port. The file is sent via the Zmodem Protocol.

Result Values:

- –OK

-1      -Invalid COM Port or COM Port is unavailable/busy

-6      -No Response

-7      -File not Found

Ex.  ret=ID1.commZmodemTxFile (2 , ”c:\autoexec.bat”); //OCX version

ret= commZmodemTxFile (2 , ”c:\autoexec.bat”); //DLL version

- 4. {Long}commgetTxCount ( {Short} Portno );

-this method / API is used for retrieving the number of bytes of data in the output buffer of the specified COM port. It is usually used hand by hand with “commtxblk” to retrieve the number of bytes in the output buffer then send out  the data whenever the buffer is empty. The method / API itself has the number of bytes as its return value.

Ex.  No_of_bytes=ID1.commgetTxCount(2); //OCX version

No_of_bytes=commgetTxCount(2); //DLL version

- 5. {Long }commflushtx ( {Short}Portno );

-this method / API is used for flushing/ purging/ clearing all data in the output buffer of the specified COM port.

Ex.  ret=ID1.commflushrx (2); //OCX version

ret= commflushrx(2); //DLL version

- Receiving/Getting Data Methods or Functions:

***************************************NOTE*****************************************

- In order to use this methods properly you must first open the COM port and initialize it using “commopen” and “comminit”.

**************************************************************************************

- 1. {String}commrxblk ( {Short} Portno, {Long} Length, {Short} Timeout );

-this method / API is used for reading data from the specified COM port. The length or number of bytes to read is specified together with reading duration or timeout duration (this is specified in seconds). The method / API itself has the “Read Data” as its return value.

Ex.  Data_read=ID1.commrxblk(2, 20, 3); //OCX version

Data_read=commrxblk(2, 20, 3); //DLL version

- 2. {Long}commKermitRxFile ( {Short} Portno, {String} Path );

-this method / API is used for receiving a file from a remote PC using the Kermit Protocol. The file received will be saved at the specified path.

Result Values:

- 0      –OK

-1     -Invalid COM Port or COM Port is unavailable/busy

-2     -Bad Packet

-3     -Bad Sequence

-4     -Error on Transmission

-5     -Packet end error

Ex.  ret=ID1.commKermitRxFile(2, “c:\”); //OCX version

ret=commKermitRxFile(2, “c:\”); //DLL version

- 3. {Long}commZmodemRxFile ( {Short} Portno, {String} Path );

-this method / API is used for receiving a file from a remote PC using the Zmodem Protocol. The file received will be saved at the specified path.

Result Values:

- –OK

-1      -Invalid COM Port or COM Port is unavailable/busy

-6      -No Response

Ex.  ret=ID1.commZmodemRxFile(2, “c:\”); //OCX version

ret=commZmodemRxFile(2, “c:\”); //DLL version

- 3. {String}commpeek ( {Short} Portno );

-this method / API is used for peeking 1 byte or character from the specified port. The peeked data will not be removed from the input buffer. To read and remove the data from the input, you must use “commrxblk”.

The method / API itself has the data as its return value.

Ex.  Peeked_data=ID1.commpeek(2); //OCX version

Peeked_data=commpeek(2); //DLL version

- 4. {Long}commgetRxCount ( {Short} Portno );

-this method / API is used for retrieving the number of bytes of data in the input buffer of the specified COM port. It is usually used hand by hand with “commrxblk” to retrieve the number of bytes in the input buffer then read the data. The method / API itself has the number of bytes as its return value.

Ex.  No_of_bytes=ID1.commgetRxCount(2); //OCX version

No_of_bytes=commgetRxCount(2); //DLL version

- 5. {Long }commflushrx ( {Short}Portno );

-this method / API is used for flushing/ purging/ clearing all data in the input buffer of the specified COM port.

Ex.  ret=ID1.commflushrx (2); //OCX version

ret= commflushrx(2); //DLL version

- Miscellaneous Methods or Functions:

***************************************NOTE*****************************************

- In order to use this methods properly you must first open the COM port and initialize it using “commopen” and “comminit”.

*************************************************************************************

- 1. {Long }commbreak ( {Short}Portno, {Short}Signal );

-this method / API is used for  setting the Break signal of the specified COM port to either active or inactive. The possible values are:

For Signal: 1 (Active) or  0 (Inactive)

Ex.  ret=ID1.commbreak(2, 0); //OCX version

ret=commbreak(2, 0); //DLL version

- 2. {Long }commgetTimeout ( {Short}Portno );

-this method / API is used for getting the presetted timeout duration(in milliseconds) for sending data on the specified COM port

Ex.  Time_out=ID1.commgetTimeout(2); //OCX version

Time_out=commgetTimeout(2); //DLL version

- 3. {Long }commsetTimeout ( {Short}Portno, {Long}Timeout (in milliseconds));

-this method / API is used for setting the timeout duration(in milliseconds) on sending data of the specified COM port

Ex.  Time_out=ID1.commgetTimeout(2); //OCX version

Time_out=commgetTimeout(2); //DLL version

- 4. {Long }commgetbaud ( {Short}Portno );

-this method / API will return the current Baud rate of the specified COM port.

Ex.  Baud_rate=ID1.commgetbaud(2); //OCX version

Baud_rate=commgetbaud(2); //DLL version

- 5. {Long }commsetbaud ( {Short}Portno, {Long}Baud );

-this method / API is used for setting the Baud rate of the specified COM port.

Ex.  ret=ID1.commsetbaud(2, 9600); //OCX version

ret=commsetbaud(2, 9600); //DLL version

- 6. {Long }commgetflow ( {Short}Portno );

-this method / API will return the current flow control setting(in bitwise) of the specified COM port. The return values are:

D3 D2 D1 D0

x    x    x    1    = Xon/Xoff

x    x    1    x    = RTS/CTS

x    1    x    x    = DTR/DSR

Ex.  Flow_control=ID1.commgetflow(2); //OCX version

Flow_control=commgetflow(2); //DLL version

- 7. {Long }commsetflow ( {Short}Portno, {Short}Mode );

-this method / API will set the flow control setting(in bitwise) to be used for communication in  the specified COM port. The setting of the bitwise  values are as such:

D3 D2 D1 D0

x    x    x    1    = Xon/Xoff

x    x    1    x    = RTS/CTS

x    1    x    x    = DTR/DSR

Ex.  ret=ID1.commsetflow(2, 1); //OCX version

ret=commsetflow(2, 1); //DLL version

- 8. {Long }commgetstat ( {Short}Portno );

-this method / API will return the current status of Modem signals(in bitwise) of the specified COM port. The return values are:

D3 D2 D1 D0

x    x    x    1    = CTS is On/ Active/ Detected

x    x    1    x    = DSR is On/ Active/ Detected

x    1    x    x    = Ring is On/ Active/ Detected

1    x    x    x    = RLSD is On/ Active/ Detected

Ex.  Modem_status=ID1.commgetstat(2); //OCX version

Modem_status=commgetstat(2); //DLL version

- 9. {Long }commsetDTR ( {Short}Portno, {Short}Flag );

-this method / API is used for activating/ setting or deactivating/ clearing the DTR Modem signal of the specified COM port.

The values for the Flag are as such:

0-Deactivate/ Clear DTR signal

1-Activate/ Set DTR signal

Ex.  ret=ID1.commsetDTR(2, 1); //OCX version

ret=commsetDTR(2, 1); //DLL version

- 10. {Long }commsetRTS ( {Short}Portno, {Short}Flag );

-this method / API is used for activating/ setting or deactivating/ clearing the RTS Modem signal of the specified COM port.

The values for the Flag are as such:

0-Deactivate/ Clear RTS signal

1-Activate/ Set RTS signal

Ex.  ret=ID1.commsetRTS(2, 1); //OCX version

ret=commsetRTS(2, 1); //DLL version

- 11. {Long }commsetRxBuffer ( {Short}Portno, {Long}Size );

-this method / API is used for setting the size  of the receive/input buffer of the specified COM port.

Ex.  ret=ID1.commsetRxBuffer(2, 4000); //OCX version

ret=commsetRxBuffer(2, 4000); //DLL version

- 12. {Long }commsetTxBuffer ( {Short}Portno, {Long}Size );

-this method / API is used for setting the size  of the send/output buffer of the specified COM port.

Ex.  ret=ID1.commsetTxBuffer(2, 4000); //OCX version

ret=commsetTxBuffer(2, 4000); //DLL version

- 13. {Long }commgetinfo ( {Short}Portno );

-this method / API will return the current settings of the specified COM port. The settings returned are Baud rate, Parity, Byte size and Stop bits in bitwise/hex format. The bitwise/hex values are as such:

D3 D2 D1 D0 - HEX - Baud Rate

0    0    0    1    - 0x01 - 110

0    0    1    0    - 0x02 - 300

0    0    1    1    - 0x03 - 600

0    1    0    0    - 0x04 - 1200

0    1    0    1    - 0x05 - 2400

0    1    1    0    - 0x06 - 4800

0    1    1    1    - 0x07 - 9600

1    0    0    0    - 0x08 - 14400

1    0    0    1    - 0x09 - 19200

1    0    1    0    - 0x0a - 38400

1    0    1    1    - 0x0b - 56000

1    1    0    0    - 0x0c - 115200

1    1    0    1    - 0x0d - 256000

1    1    1    0    - 0x0e - 460800

D7 D6 D5 D4 - HEX  –  Parity

0    0    0    1    - 0x10 – Even Parity

0    0    1    0    - 0x20 – Mark Parity

0    0    1    1    - 0x30 – No Parity

0    1    0    0    - 0x40 – Odd Parity

0    1    0    1    - 0x50 – Space Parity

D11D10D9 D8 - HEX  – Bytesize

0    0    0    1    - 0x100 – 5

0    0    1    0    - 0x200 – 6

0    0    1    1    - 0x300 – 7

0    1    0    0    - 0x400 – 8

D15D14D13D12- HEX  – Stopbits

0    0    0    1    - 0x100  – 1 stop bit

0    0    1    0    - 0x200  – 1.5 stop bits

0    0    1    1    - 0x300  – 2 stop bits

Ex.  Settings=ID1.commgetinfo(2); //OCX version

Settings=commgetinfo(2); //DLL version

- Event Mapping:

***************************************NOTE*****************************************

- In order to use this methods properly you must first open the COM port and initialize it using “commopen” and “comminit”.

*************************************************************************************

- 1. {Long }commTranslateEvent ( {Short}Portno, {String}Event, {Short}Message );

-this method / API is used to specify the Message that will be triggered by the specified event. The OCX/ DLL will call the specified Message whenever the specified Event is triggered. The 1st Parameter of the Message will contain the COM port number that triggered the Event. The OCX/DLL will detect the specified event then Post the Message specifying  NULL as the window handle. The Events where Messages can be assigned are as such:

Events:        Meaning:

“FRAME”- for frame error

“IOE”- for device I/O communication error

“OVERRUN”- for buffer overrun (next character is lost)

“RXOVER”- for receive/input buffer overflow

“RXPARITY”- for receive/input parity error

“TXFULL”- for send/output buffer full (no more space in the Tx buffer)

Ex.  ret=ID1.commTranslateEvent(2, “FRAME”,  MY_MESSAGE_FRAME_ERROR); //OCX version

ret=commTranslateEvent(2, “FRAME”, MY_MESSAGE_FRAME_ERROR); //DLL version

- Sample Program Excerpts:

- Sample program to Transmit some data out to COM2 (where a modem is attached) and wait until the modem responds back to COM2.

int  Test_Modem(void)

{

long ret,length;

short COM_port;

CString data_out, data_in;

COM_port=2;

if (ID1.commopen (COM_port)= = -1) return -1; //cannot open COM port

ret=ID1.flushtx(COM_port);

ret=ID1.flushrx(COM_port);

ret=ID1.comminit (COM_port, 9600, 8, “NO”, 1);

ret=ID1.commsetRTS(COM_port, 0x01);

data_out=”AT”;

data_out+=0x0d;

length=data_out.GetLength( );

ret=ID1.commtxblk(COM_port, data_out, length);

while (commgetRxCount(COM_port) = = 0)

{

Sleep(0);

}

data_in=””;

data_in=ID1.commrxblk(COM_port, 10, 10);

MessageBox(data_in);

ret=ID1.commclose(COM_port);

}