CHAPTER 1

INTRODUCTION

The  PCCOM V2.0 & V5.1 is a  high  performance,  easy  to  use RS232/RS422 device  driver  for PC/XT, PC/AT, PC/386, PC/486 or  compatibles. Under MS/DOS environment, you can set up your serial ports by PCCOM  device  driver,  and these  serial ports can be treated  as  COM1: and   COM2:  devices.  The  setup   procedure provides  flexible functions to  specify  the configuration of multi-serial card, that  is, the  hardware  configurations  of  I/O   port number,  I/O  port  address,  interrupt   and interrupt vector are user selectable.

After the device driver is installed, It takes over communication between CPU and multi-serial cards such as four port card, eight port card, ... etc. For each I/O port, the service routine handles a ring buffer to keep track of all I/O data.  Moreover, the PCCOM provides library routines (C, PASCAL, BASIC, FoxPro) and   DOS    communication

interface (DOS device driver, BIOS call)  for several access levels.

The PCCOM V2.0 is an upgrade version of PCCOM V1.0 software it combines with PCCOM V1.0 and SERIAL DRIVER utilities.  While PCCOM V5.1 is on upgrade version of PCCOM V2.0 software, that supports both ISA & PCI series cards.  Therefore, PCCOM V2.0 & V5.1 device driver supports up to 64 ports and each serial port may be either 8250, 16450, or 16550 (FIFO) chip.

The features of PCCOM are:

- Support RS232/RS422 device driver under MS/DOS.

- I/O  port  number,   I/O   port   address, interrupt,  interrupt vector,  and   ring buffer size selectable.

- Communication parameters  (baud   rate, parity, ... etc. ) selectable.

- Device name is specified by the user and it can be treated as a file to perform I/O operations.

- The  device driver start up by  directives of CONFIG.SYS file.

- Up  to  64 ports can be  driven  and  each serial port may be either 8250, 16450,  or 16550  (FIFO) chip which are  detected  by PCCOM automatically.

- Support C, PASCAL, and BASIC, FoxPro language library routines to link user's program.

- Support RS232/RS422 diagnostic programs, file   transfer,  and   virtual   terminal evaluator software.

- Virtual   terminal    evaluator   provides graphics capability.

Comparisons between PCCOM V2.0 and PCCOM V1.0 plus SERIAL DRIVER:

- Upgrade   version   of PCCOM V1.0    and combines with PCCOM V1.0 plus   SERIAL    DRIVER.

- New installation procedure. (need not make serial driver).

- Provides DOS device driver features which    are COM1: and COM2: like.

- Supports 8250, 16450, and 16550 drivers.

- More than 8 ports use common IRQ (up to 24 ports).

- More than 8 ports in one device driver (up to 64 ports).

- More than 8K bytes ring buffer is used in one port (up to 32K bytes).

- Support ISA bus communication interface  ( it means IRQ select can be 2, 3, 4, 5,  6, 7, 10, 11, 12, 14, 15).

- Provides both BIOS call and DOS device driver interface.

- Provides C, PASCAL, BASIC, FOXPRO libraries.

Comparison between PCCOM V5.1 and PCCOM V2.0

- Supports both ISA and PCI series cards.

CHAPTER 2

CHECK YOUR SYSTEM

2.1 Check List

Before getting started, check that your PCCOM package includes the following items:

- One diskette, which   contains   PCCOM software.

- PCCOM user's manual.

2.2 System Requirements

Before installing your PCCOM package, you need to make sure that the   system configuration as follows:

- IBM  PC/XT,  PC/AT,  PC/386,  PC/486, Pentium or   compatibles.

- MS/DOS V3.1 operating system or above.

- At least 1M bytes main memory.

- At least one serial port available.

2.3 Distribution Disk Contents

The distribution disk   contains PCCOM device drivers and virtual terminal software, which are shown in the following:

PCCOM.SYS

The PCCOM device driver.

PCCOM.OPT

Hardware configuration of PCCOM startup file.

SETUP.EXE

The  SETUP  program  is  used  to  setup communication

protocol.

MODESET.EXE

Report the status or reset some flags of specified port.

VT.EXE

VT52/VT100 virtual terminal evaluator.

TESTCOM.EXE

The serial port diagnostic program.

RX.EXE

File transfer command.  The RX receive data from remote

computer.

TX.EXE

File transfer command.  The TX transmit data to remote

computer or terminal.

COMLIB.H

Header file of c library.

COMLIBBC.C  COMLIBMS.C

Communication    library   routines   to control PCCOM device driver. The library is written by Borland C and MSC language.

COMLIBV2.PAS

Communication   library   routines    to control PCCOM device driver. The library is written by  PASCAL  language.

COMLIBGW.BAS  COMLIBQK.BAS   COMLIBQ.BAS

COMLIBTB.BAS

Communication   library   routines    to control PCCOM device driver. The library  is written by GWBASIC, Quick BASIC, Q BASIC, and Turbo BASIC.

COMLIBFP.LIB

Communication   library   routines    to control  PCCOM device driver. The library  is written by FoxPro.

TESTCOM.CFG

The   configuration   file of   TESTCOM program.

VT.CFG

The configuration file of VT program.

RSETUP.C

The source program of SETUP program.

RRX.C

The source program of RX program.

RTX.C

The source program of TX program.

LOOPBACK.BAS

Loopback test.

DTX.BAS

Transfer file.

DRX.BAS

Receive file.

LOOPBACK.C

Loopback test.

DTERM.C

ASCII terminal emulator.

DCLOCK.C

Receive clock message.

LOOPBACK.PAS

Loopback test.

DTERM.PAS

ASCII terminal emulator.

*.CFG

Configuration files of TESTCOM program.

*.OPT

Configuration files of PCCOM.SYS.

README.DOC

Release note.

2.4 Backup Your Diskette

Before you begin installing your PCCOM, you need to have working copies of PCCOM software diskette you received.  This will help preserve your original PCCOM diskette. Copy the PCCOM diskette, using the   DOS DISKCOPY command. Please see your DOS manual if necessary.

2.5 Some Restrictions

The PCCOM uses INT 1C interrupt, it was inserted in INT 1C structure, does not effect normal INT operation.

CHAPTER 3

PC COM INSTALLATION

The PCCOM.SYS is used to install device drivers for serial communication port.  User may specifies I/O port address, baud rate, interrupt vector...etc. for different hardware   configuration.  To   setup   PCCOM complete the following steps:

STEP 1:  Insert your serial communication adapter and

Connect communication cable.

STEP  2:  Start the computer.

STEP  3:  Use PCCOM.SYS  program to install device

Driver and setup different communication protocol.

You must insert PCCOM.SYS commands to the CONFIG.SYS file, which will   install device driver automatically when computer is booted.

3.1 Device Driver Installation

The PCCOM.SYS program is used to install device driver.   To install PCCOM   device driver, please write the following command format into CONFIG.SYS file.

DEVICE = PCCOM.SYS @filename

@ - Prefix  symbol to  specify  configuration file.

Filename - Configuration file, which is used to specify hardware configuration of the system.

EXAMPLE 3.1.1

DEVICE = PCCOM.SYS @c:\pccom.opt

Set up device driver, which is specified by c:\pccom.opt file.

3.2 Hardware Configuration Specification

To specify hardware configuration, you must follow the PCCOM set up syntax. In this section, we only introduce how to write the hardware configuration file, more details syntax specification please see the APPENDIX C. In the hardware configuration file, three directives must be used.

/B:portno   /D:devicename  /A:setup

/B:portno

Specify the first port number.

/D:devicename

Specify the first device name.

/A:setup

Set up communication parameters and hardware    configuration for ISA series cards.

/P:  setup

For ISA series cards.

/P: setup

Set up communication parameters and hardware configuration for PCI series cards.

The syntax is:

/P: [card ID: PortNo: Port]

Most important thing is to describe /A: directive for ISA series cards and the /P: directive for PCI series cards.  now let’s introduce   in   the following.  In order to familiar how to set up, we give some examples first.

EXAMPLE 3.2.1

Suppose we install one 4 port card, its port no. is defined from 4, its device name is defined from COM5. Hardware configuration is specified as:

Port no. = 4, 5, 6, 7

Device name = COM5, COM6, COM7, COM8

Interrupt request = 5

Port address = 2A0, 2A8, 2B0, 2B8

Interrupt vector = 2BF

Interrupt vector action = Active LOW

Communication parameters

| port no. | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- |
| port address | 2A0 | 2A8 | 2B0 | 2B8 |
| ring buffer | 1K | 512 | 256 | 128 |
| baud rate | 9600 | 9600 | 9600 | 9600 |
| parity | none | none | none | none |
| data bit | 8 | 8 | 8 | 8 |
| parity bit | 1 | 1 | 1 | 1 |
| modem control | DTR RTS OUT2 | DTR RTS OUT1 | DTR RTS | NONE |
| flow control | XON | XON | XON | XON |

Then the hardware configuration file  is defined as:

#bios port no = 4, 5, 6, 7

/B:4

#dos device name = COM5, COM6, COM7, COM8

/D:COM5

/A:[5:

2A0,4,2BF,LO:

(1k :9600:N-8-1:DTR+RTS+OUT2:XON),

(512:9600:N-8-1:DTR+RTS+OUT1:XON),

(256:9600:N-8-1:DTR+RTS:XON),

(128:9600:N-8-1::XON)]

Where # is comment. Each IRQ control one set of ports and was enclosed by  [   ]  block. Communication parameters for each port is enclosed by (  ) block.

EXAMPLE 3.2.2

Suppose  we  install one  8  port  card, hardware configuration is specified as:

Port no. = 0, 1, 2, 3, 4, 5, 6, 7

Device name = COM5, COM10, COM11, COM12

COM13, COM14, COM15, COM16

Interrupt request = 5

Port address = 280, 288, 290, 298,

2A0, 2A8, 2B0, 2B8

Interrupt vector = 2C2

Interrupt vector action = Active LOW

##### Communication parameters

| port no. | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| port address | 280 | 288 | 290 | 298 |
| ring buffer | 1K | 1k | 1k | 1k |
| baud rate | 4800 | 4800 | 4800 | 4800 |
| parity | even | even | even | even |
| data bit | 7 | 7 | 7 | 7 |
| parity bit | 2 | 2 | 2 | 2 |
| modem control | DTR RTS | DT RTS | DTR RTS | DTR RTS |
| flow control | DTR | DTR | DTR | DTR |

| port no. | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- |
| port address | 2A0 | 2A8 | 2B0 | 2B8 |
| ring buffer | 2K | 2k | 2k | 2k |
| baud rate | 4800 | 4800 | 4800 | 4800 |
| parity | none | none | none | none |
| data bit | 8 | 8 | 8 | 8 |
| parity bit | 1 | 1 | 1 | 1 |
| modem control | DTR RTS | DTR RTS | DTR RTS | DTR RTS |
| flow control | XON | XON | XON | XON |

Then the hardware configuration file  is defined as:

#bios port no = 0, 1, 2, 3, 4, 5, 6, 7

/B:0

#dos device name = COM5,  COM10, COM11, COM12

#                  COM13  COM14, COM15, COM16

/D:COM5, COM10

/A:[5:

280,8,2C2,LO:

(1k:4800:E-7-2:DTR+RTS:DTR) * 4,

(2k:4800:N-8-1:DTR+RTS:XON) * 4]

EXAMPLE 3.2.3

Suppose we install 16 ports, hardware configuration is specified as

Port no. = 8, 9, 10, 11, 12, 13, 14, 15,

16,  17, 18, 19, 20, 21,  22,

23

Device name = COM1, COM3, AAA2, AAA3,

AAA4, AAA5, AAA6, AAA7,

AAA8, AAA9, AAA10, AAA11,

AAA12, AAA13, AAA14, AAA15

Interrupt request = 15

Port address = 280, 288, 290, 298,

2A0, 2A8, 2B0, 2B8,

180, 188, 190, 198,

1A0, 1A8, 1B0, 1B8,

Interrupt vector = 2C2, 1C2

Interrupt vector action = Active LOW

Communication parameters

| port no. | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- |
| port address | 280 | 288 | 290 | 298 |
| ring buffer | 1K | 1k | 1k | 1k |
| baud rate | 2400 | 2400 | 2400 | 2400 |
| parity | none | none | none | none |
| data bit | 8 | 8 | 8 | 8 |
| parity bit | 1 | 1 | 1 | 1 |
| modem control | DTR RTS | DTR RTS | DTR RTS | DTR RTS |
| flow control | DTR | DTR | DTR | DTR |

| port number | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- |
| port address | 2A0 | 2A8 | 2B0 | 2B8 |
| ring buffer | 1K | 1k | 1k | 1k |
| baud rate | 2400 | 2400 | 2400 | 2400 |
| Parity | none | none | none | none |
| data bit | 8 | 8 | 8 | 8 |
| parity bit | 1 | 1 | 1 | 1 |
| modem control | DTR RTS | DTR RTS | DTR RTS | DTR RTS |
| flow control | DTR | DTR | DTR | DTR |

Port  16  to  port 23  are set  to  1K  bytes buffer,  baud  rate is 2400, none  parity,  8 data bits, 1 stop bit, modem control flow are DTR+RTS,  flow control is XON, and  its  port address  start  from 180  to  1B8,  interrupt vector is 1C2, which was active low.

Then the hardware configuration file is defined as:

#bios port no= 8,  9, 10, 11, 12, 13, 14, 15,

#             16, 17, 18  19, 20, 21, 22, 23

/B:8

#dos device name= COM1, COM3, AAA2,  AAA3,

#                 AAA4, AAA5, AAA6, AAA7,

#                 AAA8, AAA9, AAA10, AAA11,

#                 AAA12, AAA13, AAA14, AAA15

/D:COM1, COM3, AAA2

/A:[15:

280,8,2C2,LO:

(1k:2400:N-8-1:DTR+RTS:RTS),

(1k:2400:N-8-1:DTR+RTS:RTS),

(1k:2400:N-8-1:DTR+RTS:RTS),

(1k:2400:N-8-1:DTR+RTS:RTS),

(1k:2400:N-8-1:DTR+RTS:DTR) * 4 :

180,8,1C2,LO:

(1k:2400:N-8-1:DTR+RTS:XON) * 8]

EXAMPLE 3.2.4

Suppose we install 16 ports, hardware configuration is specified as

Port no. = 8, 9, 10, 11, 12, 13, 14, 15,

16,  17, 18, 19, 20, 21,  22, 23

Device name = COM1, COM3, AAA2, AAA3,

AAA4, AAA5, AAA6, AAA7,

AAA8, AAA9, AAA10, AAA11,

AAA12, AAA13, AAA14, AAA15

Interrupt request = 5,7

Port address = 280, 288, 290, 298,

2A0, 2A8, 2B0, 2B8,

180, 188, 190, 198,

1A0, 1A8, 1B0, 1B8,

Interrupt vector = 2C2, 1C2

Interrupt vector action = Active LOW

Port  8  to  port 15  are  set  to  1K  bytes buffer, baud rate is 1200, non parity, 8 data bits,  1  stop bit, modem  control  flow  are DTR+RTS,  flow control is XON, and  its  port address  start from 280 to 2B8, interrupt  is 5, interrupt vector is 2C2, which was  active low.

Port 16 to port 23 are set  to  1K  bytes buffer, baud rate is 1200, non parity, 8 data bits,  1  stop bit, modem  control  flow  are DTR+RTS,  flow control is XON, and  its  port address  start from 180 to 1B8, interrupt  is 7, interrupt vector is 1C2, which was  active low.

Then the hardware configuration file is defined as:

#bios port no= 8,  9, 10, 11, 12, 13, 14, 15,

#             16, 17, 18  19, 20, 21, 22, 23

/B:8

#dos device name= COM1, COM3, AAA2,  AAA3,

#                 AAA4, AAA5, AAA6, AAA7,

#                 AAA8, AAA9, AAA10, AAA11,

#                 AAA12, AAA13, AAA14, AAA15

/D:COM1, COM3, AAA2

/A:[5:

280,8,2C2,LO:

(1k:1200:N-8-1:DTR+RTS:XON) * 8],

[7:

180,8,1C2,LO:

(1k:1200:N-8-1:DTR+RTS:XON) * 8]

EXAMPLE 3.2.5

Install standard COM1 and COM2.

/B:0

/D:COM1

/A:[4:

3F8,1,0,HI:

(8k:9600:N-8-1:DTR+RTS+OUT2:XON)],

[3:

2F8,1,0,HI:

(8k:9600:N-8-1:DTR+RTS+OUT2:XON)]

EXAMPLE 3.2.6.

Install a single 4 Port PCCOM PCI card.

/D: COM5

/B:4

/P:[:4(1K: 9600: N-8-1:: XON) * 4]

EXAMPLE 3.2.7

Install a 2 Port PCCOM PCI card with card ID of 1 and a 8 Port PCCOM PCI card with card ID of 2.

/D: COM3

/B: 4

/P: [1:2:(1k:9600:N-8-1:: XON)*2)

/P: [2:8:(1k: 9600: N-8-1::XON)*8)

3.3 Some Constraints

In the hardware configuration file, each set owns one interrupt, which is enclosed by [] block, and each set controls either 1

port, 2 ports, 4 ports, 8 ports, 16 ports, or 24 ports, which is enclosed by (  ) block. Up to 8 sets can be set in one computer  system, and  total support up to 64 serial ports.  In the  following, we describe  some  constrains when set up communication parameters.

device name

Begin with a letter and must consist only letters  and digits. Its length can not  more than 8.

interrupt

Interrupt  will be one of 2, 3, 4, 5,  6, 7, 10, 11, 12, 14, 15.

interrupt vector

Interrupt   vector  will  be  any   legal address or set to 0 means need not  interrupt vector.

interrupt activation

Using  HI or LO to set up active high  or active low. If interrupt vector not be  used, this filed is ignore.

buffer size

The maximum size for one port  is  32KB, and  one set can not be allocated  more  than 64KB. However, the size of total buffer  used in driver can be larger than 64 KB.

baud rate

Baud  rate  can  be any  valid  value  in serial communication protocol.

parity

Either E, O, N can be selected which means even, odd, none parity.

data bit

Either  5,  6,  7, 8  data  bits  can  be selected.

stop bit

Either 1 or 2 bits can be selected.

modem control

You can choice communication protocol you need. Multiple protocols are allowed to set in on port.  DTR  (Data  Terminal  Ready),  RTS (Request  To  Send), OUT1, and  OUT2  can  be

selected. If you ignore this field, it means you need not modem control.

flow control

Multiple   protocol   are   allowed.   It supports  hardware handshaking which are  DTR and RTS, and software handshaking XON. If you ignore this field, it means you need not flow control.

#

Begin   in  column  one  to  specify   as comments.

More detail syntax specification was specified in appendix C and communication parameters were introduced in appendix B.

CHAPTER 4

RUN TIME SUPPORT

4.1 How To Use Device

After  device driver is  installed,  you may use device name  as a file to perform I/O operations.

To  send  file  to  the  COM4 port,  the command is shown in the following.

A>copy filename COM4

To receive file from the COM5 port,  the command is shown as

A>copy COM5 filename

4.2 Communication Parameters Set up

You may set up communication parameters by hardware configuration file, which will be executed when system is booted. However, you can set up communication parameters at run time   by use SETUP command.  The   syntax specification of SETUP is

SETUP [/v,]portno,[baud,parity,data,stop][,MC=ms][,FC=fs]

The  default parameters  are  baud=9600, parity=N,  data=8, stop=1, no  MODEM  control and flow control.

/v

Verbose.  List all  setting  parameters. After  serial  ports are setup,  system  will check whether the specified port is good,  if the  corresponding port is bad,  system  will prompt ???? to notice user.

portno

Specifies which port is control by PCCOM device driver.

baud

Baud rate. You may key in any baud rate, the device driver will adjust your baud  rate to desire baud rate that the serial port  can accept. If you key in 65535 baud, this  means 115200.  According  to our  test,  The  PCCOM device  driver can accept up to  115200  baud under 8m cable.

parity

Parity bit.  N : none parity,

E : even parity,

O : odd parity.

data

Data  bits.  5 :  five data bits,

6 :  six data bits,

7 :  seven data bits,

8 :  eight data bits.

stop

Stop bits.  1 :  one stop bit,

2 :  two stop bits.

MC=ms

Select Modem Control Register.  Multiple protocol  will  be selected, which  are  DTR, RTS, OUT1, OUT2. NONE means ignore.

FC=fs

Select     hardware     or      software handshaking.  Multiple protocol are  selected which  are  DTR,  RTS, and  XON.  NONE  means ignore.

The  following examples are provided  to help clarify the use of the SETUP command.

EXAMPLE 4.2.1

A>  SETUP 1,MC=DTR+RTS+OUT2

To  setup I/O port 1. It set  DTR,  RTS, and OUT2 bits of modem control register,  the default   parameters  are   baud   rate=9600, parity=N,  data  bits=8,  stop  bits=1,  flow control is none.

EXAMPLE 4.2.2

A>  SETUP /v,2,9600,E,8,2,MC=DTR+RTS,

FC=XON

To setup I/O port 2.  Its handshaking protocol is  XON/XOFF, baud  rate  is  9600, parity  check is even, data bits is  8,  stop bits is 2, modem control register is DTR+RTS. After I/O ports are setup, system lists all setting parameters.

EXAMPLE 4.2.3

A>  SETUP 0,FC=XON

To  setup  I/O port 0.  Its handshaking protocol is XON/XOFF. Baud rate =9600, parity check is none, data bits is 8, stop bits is 1, MC is none.

4.3 Setup and Status Report

You may change communication parameters, reset some communication flags or check the status of serial port by  execute  MODESET program,  which is used  interactively.  When you execute the MODESET program,  you  only need  to specify device name which you  need, then modify communication parameters by arrow key to select the corresponding field.

CHAPTER 5

FILE TRANSFER

After device driver is installed and communication parameters are setup, you may use standard device name (COM1 and COM2 like) to send and receive data to/from the port you specify.

In PCCOM, we also provide TX.EXE and RX.EXE utilities to perform  file  transfer functions, which are more flexible.

5.1 Transmit File to Remote Site

The   command  format  of   TX.EXE   is specified in the follows.

TX [/PnoLR < filename]

filename

Select which file to be transmitted  to remote site.  The < filename specification is I/O  redirection of MS/DOS.  Suppose you want key  in  characters from keyboard  to  remote site,  you  need  not specify file  name  and after   end  of  transmission,  please   type Z/CTRL.

Pno

Select which I/O port to send data. The default is P0.

L

Specify  the transmitted data are local echo on the screen.

R

Specify  the transmitted data are  echo from remote site to local screen.

NOTE

If  the  received  file  size  less  one character  than  the transmitted  file  size, don't  worry!  the TX.EXE program  eliminates one <CR> code from transmitted file.

EXAMPLE 5.1.1

Suppose  we want send COMLIBBC.C file  to remote  site  through  port  number  2.   The command is as follows.

A>  TX /P2 < COMLIBBC.C

EXAMPLE 5.1.2

To  send  RRX.C  file  to  remote  site through port number 0.

A>  TX < RRX.C

EXAMPLE 5.1.3

To  send  RRX.C  file  to  remote  site through  port  number 3 and  local  echo  the RRX.C contents on the monitor.

A>  TX /P3L < RRX.C

EXAMPLE 5.1.4

Key in data to remote site through port number 0.

A>TX

hello, this is a test procedure.

Z/CTRL

5.2 Receive Data from Remote Site

The RX.EXE is used to receive data file from  remote  site,   its  command format  is specified in the follows.

RX [/PnoE > filename]

Select  which I/O port to receive data. The default is P0.

L

Specify  the  received data  are  local echo on the screen.

R

Specify  the received data are echo  to the sending site.

Suppose you want store received data to disk file,  please use MS/DOS I/O redirection features.  To redirect your I/O, type the following command.

A>  RX /P4 >temp.dat

The  received data will be stored to temp.dat  file.

EXAMPLE 5.2.1

Suppose we want receive data   from remote  site  through  port  number  2.   The command is as follows.

A>  RX /P2

EXAMPLE 5.2.2

To   receive  data  from  remote   site through port number 0.

A>  RX

EXAMPLE 5.2.3

To  receive  data  from   remote   site through  port  number 3 and  stores  received data   to  temp.dat  file.  Local  echo   the

received data are active.

A>  RX /P3L >temp.dat

5.3 Two Way Communication

To  transfer file from one computer  to another  computer,  the communication  scheme and command are shown in the following.

TX /P2L < temp.dat                                    RX /P1L > aa.dat

There are several combinations of  local echo  and remote echo.  In the following,  we give  some examples to explain how to  use  L and R parameters.

EXAMPLE 5.3.1

Suppose   your   receiver site   is   a terminal, and transmitter site is a PC, and you want to see the transmitted data on the PC screen, then command of transmitter site is

A>  TX /P0L < TEMP.DAT

EXAMPLE 5.3.2

The  same as the EXAMPLE 5.3.1,  but you need  not display transmitted data on the  PC screen.

A>  TX < TEMP.DAT

EXAMPLE 5.3.3

Suppose   your   transmitter   site   is terminal  who has no ability to  display  the key  in data on the terminal (no  local  echo ability)  and  receiver site is PC.  In  this case  the  receiver site must  echo  received characters to the terminal. The command is

A>  RX /R > AA.DAT

EXAMPLE 5.3.4

The same as EXAMPLE 5.3.3,  but you want receiver  site display the received  data  on the screen.

A>  RX /LR > AA.DAT

EXAMPLE 5.3.5

Both  receiver and transmitter site  are PC,  and  you need not show  the  transmitted data.

A>  TX < TEMP.DAT

A>  RX > AA.DAT

EXAMPLE 5.3.6

The same as EXAMPLE 5.3.5,  but you need show the data on the both computer.

A>  TX /L < TEMP.DAT

A>  RX /L > AA.DAT

another method is

A>  TX /R < TEMP.DAT

A>  RX /LR > AA.DAT

CHAPTER 6

PROGRAMMING TECHNICAL UNDER MS DOS

There   are three methods   to   write application   programs to   control   serial communication.

1. Use device driver name to perform I/O.

2. Use    PCCOM   library     through   DOS interrupt

routines.

3. Use low-level BIOS interrupt routines.

6.1 How to Use Device Driver

After  device driver is  installed,  you may use device name (such as COM5, COM6,  ... etc.)  as a file to perform  I/O  operations. Since  many  high  level  languages   provide several  kinds of I/O operations, e.g.  in  C language,  the write statement for low  level I/O and fprintf for high level I/O. You  must care which I/O routine is used by yourself to prevent the lose of I/O data, if you use high level I/O, you must use flush to dump  buffer contents to serial device when necessary.

In  the  distribution disk,  we  provide BASIC, C, PASCAL, and FoxPro examples, all of these programs use device driver.

LOOPBACK.BAS

Loopback test.

DTX.BAS

Transfer file.

DRX.BAS

Receive file.

LOOPBACK.C

Loopback test.

DTERM.C

ASCII terminal emulator.

DCLOCK.C

Receive clock message.

LOOPBACK.PAS

Loopback test.

DTERM.PAS

ASCII terminal emulator.

6.2 PCCOM Library

PCCOM   provides  C, PASCAL,  BASIC,  and FOXPRO library routines to let user to  write application   programs  to   control   serial communication via device drivers.

To  write an  application  program,  you must link COMLI115.C or COMLI460.C (Borland C),  COMLIBMS.C (MSC),  PCCOM115.PAS or PCCOM460.PAS (PASCAL), COMLIBGW.BAS (GWBASIC),   COMLIBQK.BAS   (Quick    BASIC) or  COMLIBQ.BAS  (Q BASIC) file according to your selection language. In  the following, we describe the PCCOM library more details:

| Routine Name | Description |
| --- | --- |
| cominit | setup communication parameters |
| comtxch | send a character |
| comrxch | receive a character |
| comstat | check communication status |
| comactive | check if port is usable |
| comsetDTR | set DTR signal ON or OFF |
| compeek | check next char in queue |
| comsetflow | set flow control |
| comtxblk | send one block data |
| comrxblk | receive one block data |
| combreak | end break signal |
| cominstalled | check if driver installed |
| comsetport | set address of serial port |
| comsetvector | set interrupt vector |
| comsetactive | set active low or high |
| comsetMCR | set MCR contents |
| compurgerxbuf | remove received buffer data |
| comtxs | send a character string |
| comrxs | receive a character string |
| comsettimeout | set time out |
| comgettimeout | get time out |
| combufstat | return no. of chars in queue |
| comgetMCR | get MCR contents |
| comgetFC | get flow control |
| comgetsets | get number of PCCOM sets |
| comgetsetinfo | get PCCOM set information |
| comgetinfo | get port status |
| comgetports | get number of PCCOM ports |
| comdetectport | detect if the port exist |
| comgetportinfo | get one port information |
| comrxoff | stop send data |
| comrxon | restart send data |

1. cominit(portno, baud, db, parity, sb)

int cominit

unsigned int  baud

int  portno, db, parity, sb

The cominit is used to setup several communication parameters.

portno (int)

port    number    that    defined     in configuration file.

baud (unsigned int)

baud  rate. Either B38400, B19200, B9600, B4800, B2400, B1200, B600, B300.

| B38400 (38400) | 38400 baud. |
| --- | --- |
| B19200 (19200) | 19200 baud. |
| B9600 (9600) | 9600 baud. |
| B4800 (4800) | 4800 baud. |
| B2400 (2400) | 2400 baud. |
| B1200 (1200) | 1200 baud. |
| B600 (600) | 600 baud. |
| B300 (300) | 300 baud. |

db (int)

data  bits.  Either CS8,  CS7, CS6, CS5.

| CS8 (8) | 8 bits |
| --- | --- |
| CS7 (7) | 7 bits |
| CS6 (6) | 6 bits |
| CS5 (5) | 5 bits |

parity (int)

parity bit. Either PNONE, PEVEN, PODD.

| PNONE (0 | none parity check. |
| --- | --- |
| PODD (1 | odd parity. |
| PEVEN (2) | even parity. |

sb (int)

stop bits. Either ST1 or ST2.

| ST1 (1) | one stop bit. |
| --- | --- |
| ST2 (2) | two stop bits. |

return value (int)

0 : success.

1 : fail.

2. comtxch(portno, ch)

int comtxch

int portno

char ch

The comtxch is used to send a  character to serial port.

ch (char)

transmitted character

return value (int)

| 0 | success. |
| --- | --- |
| 1 | fail. |

3. comrxch(portno, ch)

int comrxch

int portno

char *ch

The comrxch is used to get a character.

ch (char*)

received character.

return value (int)

| 0 | success. |
| --- | --- |
| 1 | fail (receive no character). |

4. comstat(portno)

int comstat

int portno

The  comstat is used to get  serial  port status,  the status indicator is shown in the following.

return value (int)

| bit 15 | time out error. |
| --- | --- |
| bit 14 | transmitter shift register empty. |
| bit 13 | transmitter holding register empty. |
| bit 8 | data ready (data available in input buffer). |
| bit 7 | received line signal detect (RLSD). |
| bit 5 | data set ready (DSR). |
| bit 4 | clear to send (CTS). |

You  may check the parameters to  detect which bit is set.

| definition | position | value |
| --- | --- | --- |
| S_CTS | 4 | 0X0010 |
| S_DSR | 5 | 0X0020 |
| S_DCD | 7 | 0X0080 |
| S_RDA | 8 | 0X0100 |
| S_THRE | 13 | 0X2000 |
| S_TSRE | 14 | 0X4000 |
| S_TIMEOUT | 15 | 0X8000 |

5. comactive(portno)

int comactive

int portno

Check if the port is active.

return value (int)

| 1 | active |
| --- | --- |
| 0 | inactive |

6. comsetDTR(portno,n)

void comsetDTR

int portno,n

Set DTR signal ON or OFF.

n (int)

| ACTIVE (1) | set DTR ON |
| --- | --- |
| INACTIVE (0) | set DTR OFF |

7. compeek(portno)

int compeek

int portno

The   compeek   is   used   to   perform nondestructively  read a character. The  read character remains in the receive buffer.

return value (int)

| 0XFFFF | no character |
| --- | --- |
| ? | received character |

8. comsetflow(portno,n)

void comsetflow

int portno, n

To set communication protocol.

n (int)

| F_XONXOF (0X09) | set XON/XOFF protocol. |
| --- | --- |
| F_RTSCTS (0X01) | set CTS/RTS protocol. |
| F_DTRDSR (0X04) | set DSR/DTR protocol. |

9. comtxblk(portno, pointer, n)

int comtxblk

int portno,n

void *pointer

To transmit a block of data, it uses one BIOS call to transmit a block of data.

pointer (pointer)

pointer to start of block.

n (int)

block length.

return value (int)

number of characters be sent.

10. comrxblk(portno, pointer, n)

int comrxblk

int portno, n

void *pointer

To receive a block of data,  it uses one BIOS call to receive a block of data.

pointer (pointer)

pointer to start of block.

n (int)

number of characters should be received.

return value (int)

number of characters actually received.

11. combreak(portno,brk)

void combreak

int portno, brk

Send the break signal to driver.

brk (int)

| ACTIVE (1) | break the communication. |
| --- | --- |
| INACTIVE (0) | unbreak the communication |

12. cominstalled(portno)

int cominstalled

int portno

Check if the driver is installed ?

return value (int)

| 0 | is installed |
| --- | --- |
| 1 | not installed |

13. comsetport(portno, addr)

void comsetport

int portno, addr

The  comsetport  is used to  set  serial port address.

addr (int)

serial port address.

14. comsetvector(portno, addr)

void comsetvector

int portno, addr

The   comsetvector   is  used   to   set interrupt  vector  address.  After  interrupt vector is set, when interrupt is emerged then

the  device  driver  will  detect   interrupt vector  to  check  which  port  emerges   the interrupt.

addr (int)

interrupt vector address. The address is designed at hardware adapter.

15. comsetactive(portno, active)

void comsetactive

int portno

char active

The   comsetactive   is  used   to   set interrupt vector is active low or high.

active (char)

| 0 | active low. |
| --- | --- |
| 1 | active high |

16. comsetMCR(portno, mcr)

void comsetMCR

int portno, mcr

The  comsetMCR  is used to  control  the contents  of modem control  register.  Please see APPENDIX B for more details.

mcr (int)

| bit 7 | 0 |
| --- | --- |
| bit 6 | 0 |
| bit 5 | 0 |
| bit 4 | loopback. |
| bit 3 | OUT2 signal |
| bit 2 | OUT1 signal |
| bit 1 | RTS signal |
| bit 0 | DTR signal |

.

17. compurgerxbuf(portno)

void compurgerxbuf

int portno

Remove  any  received buffer  data  from specify port.

18. comtxs(portno, s)

int comtxs, portno

char *s

The comtxs is used to send one string to serial   port  by  calling  BIOS   for   each character.

s (char*)

one character string to be sent.

return value (int)

| 0 | success |
| --- | --- |
| 1 | fail |

19. comrxs(portno, s)

int comrxs, portno

char *s

The  comrxs  is used to get  one  string from  ring  buffer by calling BIOS  for  each character. This routine gets characters  from ring buffer until it find <CR> or ring buffer is empty.

s (char*)

received string.

return value (int)

| 0 | fail |
| --- | --- |
| 1 | Success |

20. comsettimeout(portno,timeout)

void comsettimeout

int portno

unsigned int timeout

Set  time  out  value  about  tx;   when transmitter  buffer in the specified port  is full, the driver will wait until buffer empty or the timeout value is expired.

timeout (unsigned int)

time out value, the unit is 1/18 sec.

21. comgettimeout(portno)

unsigned int comgettimeout

int portno

Get the tx timeout value.

return value (unsigned int)

The time out value, unit is 1/18 sec.

22. combufstat(portno)

int combufstat

int portno

The  combufstat  is used  to  count  the received characters in buffer.

return value (int)

| 0xFFFF | Buffer overflow |
| --- | --- |
| otherwise | Number of characters in received buffer |

23. comgetMCR(portno)

char comgetMCR

int portno

Get the value of MCR.

return value (char)

modem control value (see comsetMCR).

24. comgetFC(portno)

char comgetFC

int portno

Get flow control value.

return value (int)

Flow control value (see comsetflow).

25. comgetsets()

int comgetsets

Get  number  of  PCCOM  set  which   are controlled by device driver.

return value (int)

Number of PCCOM set.

26. comgetsetinfo(portno)

BiosComSet_t *comgetsetinfo

int portno

To get PCCOM set information   about specified port, which belongs to this set.

return value (BiosComSet_t*)

The set information, which is defined as

typedef struct {

unsigned char irq;

unsigned int portno;

unsigned int groupno;

unsigned int whichgroup;

struct {

unsigned int base;

unsigned int vector;

unsigned char active;

unsigned char portno;

} group[8];

} BiosComSet_t;

Where

| irq | irq number of this set. |
| --- | --- |
| portno | number of serial ports in this set. |
| groupno | number of groups in this set. |
| whichgroup | the specified belongs to which group |
| base | base I/O port address. |
| vector | interrupt vector address. |
| active | active status. |
| port | number of ports in this group. |

27. comgetinfo(portno, baud, db, parity, sb)

void comgetinfo

unsigned int *baud

int portno, *db, *parity, *sb

The   comgetinfo   is   used   to   get communication parameters of specified port.

baud (unsigned int*)

baud rate.

db (int*)

data bits. Either 5, 6, 7 or 8 data bits.

parity (int*)

parity bit. Either NONE, EVEN, ODD.

| NONE (0) | none parity check. |
| --- | --- |
| ODD (1) | odd parity. |
| EVEN (2) | even parity. |

sb (int*)

stop bits. Either 1 or 2 stop bits.

28. comgetports()

int comgetports

Get the number of PCCOM ports controlled by the PCCOM driver.

return value (int)

The number of PCCOM ports.

29. comdetectport(portno)

int comdetectport

int portno

The comdetectport is used to detect  the serial port is exist or not.

return value (int)

| 0 | the port is bad. |
| --- | --- |
| 1 | the port is ok. |

30. comgetportinfo(portno)

BiosComport_t *comgetportinfo

int portno

Retrieve   all  information  about   one specified PCCOM port.

return value (BiosComport_t*)

The port information which is defined as

typedef struct {

unsigned int addr;

unsigned int baud;

unsigned char lcr;

unsigned char mcr;

unsigned char fc;

unsigned char rx_qsz;

unsigned char rx_qfree;

unsigned char rx_enable;

unsigned char tx_enable;

} BiosComport_t;

Where

| addr | port address. |
| --- | --- |
| baud | baud rate. |
| lcr | line control register value. |
| mcr | modem control register value. |
| fc | flow control protocol. |
| rx_qsz | received ring buffer size. |
| rx_qfree | free space in rx ring buffer. |
| rx_enable | flag to notify the port receivable. |
| tx_enable | flag to notify the port transmission. |

31. comrxoff(portno)

void comrxoff

int portno

Notify the peer to stop sending data.

32. comrxon(portno)

void comrxon

int portno

Notify the peer to restart sending data.

6.3 Examples

In  the  distribution disk,  we  provide file transfer and setup program, all of these programs use PCCOM library.

RSETUP.C

Setup communication parameters.

RTX.C

Transfer file.

RRX.C

Receive file.

6.4 I/O Control Via Software Interrupt

To  communicate  directly  with   device drivers,   the  PCCOM  provides  a   set   of sub-functions  under extended BIOS  INT  14H, and  DOS IOCTL INT 21H.  All these  functions are  included in the PCCOM library,  however, if   you  want  control  I/O   through   BIOS interrupt  by yourself, you need prepare  CPU register contents and fork a system call.  In the  following,  we will describe  these  I/O control functions and teach users to use it.

All  BIOS  functions are designed  as  a FOSSIL  driver, the specification  of  FOSSIL driver is shown in the "FOSSIL.DOC" file. Our PCCOM BIOS functions specification are  shown in the "PCCOM.SPC" file. The examples of  DOS IOCTL   function  calls  are  shown  in   the

"COMDOS.C" file.

Before you perform system call, you must specify sub-function number in ah register.

| Ah | ##### Description |
| --- | --- |
| 00H | setup communication parameters |
| 01H | send one character |
| 02H | receive one character |
| 03H | get serial port status |
| 04H | activate port |
| 05H | deactivate port |
| 06H | set DTR |
| 07H | return time tick information |
| 0AH | purge input buffer |
| 0BH | send one character with no wait |
| 0CH | nondestructive read ahead |
| 0FH | flow control for serial I/O |
| 18H | block read |
| 19H | block write |
| 1AH | break begin or end |
| 1BH | return information about this driver |
| 30H | get buffer status |
| 31H | set port address |
| 32H | set interrupt vector address |
| 3AH | get the number of PCCOM sets |
| 3BH | check port exist |
| 3CH | get port information |
| 3DH | stop sending data |
| 3EH | restart sending data |
| 3FH | set time out |
| 40H | get time out value |
| 44H | IOCTL input/output functions (INT 21H) |

1. set up communication parameters

ah register

always set to 0h.

al register

al   is  used  to  setup   communication parameters.

bit 7, 6, 5 : setup baud rate.

bit 4, 3    : setup parity.

bit 2       : setup stop bits.

bit 1, 0    : setup data bits.

| baut rate | bit 7 | bit 6 | bit 5 |
| --- | --- | --- | --- |
| 9600 | 1 | 1 | 1 |
| 4800 | 1 | 1 | 0 |
| 2400 | 1 | 0 | 1 |
| 1200 | 1 | 0 | 0 |
| 600 | 0 | 1 | 1 |
| 300 | 0 | 1 | 0 |
| 38400 | 0 | 0 | 1 |
| 19200 | 0 | 0 | 0 |

| parity | 4 | 3 |
| --- | --- | --- |
| even | 1 | 1 |
| odd | 0 | 1 |
| none | x | 0 |

| stop bit | bit 2 |
| --- | --- |
| 2 | 1 |
| 1 | 0 |

| data bit | bit1 | bit 0 |
| --- | --- | --- |
| 8 | 1 | 1 |
| 7 | 1 | 0 |
| 6 | 0 | 1 |
| 5 | 0 | 0 |

dx register

port number.

return value (ax register)

bit 15 to bit 8 : line status.

bit 7 to bit 0  : modem status.

| bit 15 | time out error |
| --- | --- |
| bit 14 | transmitter shift register empty |
| bit 13 | transmitter holding register empty |
| bit 12 | 0 |
| bit 11 | 0 |
| bit 10 | 0 |
| bit 9 | Input buffer overrun |
| bit 8 | data available in receive buffer |
| bit 7 | data carrier detect |
| bit 6 | ring indicator |
| bit 5 | data set ready |
| bit 4 | clear to send. |
| bit 3 | always set to 1 upon return |
| bit 2 | delta data carrier detect |
| bit 1 | delta DSR |
| bit 0 | delta CTS |

2. send a character

ah register

always set to 1h.

al register

character.

dx register

port number.

return value (ax register)

line status and modem status.

3. receive a character

ah register

always set to 2h.

al register

received character.

dx register

port number.

return value

ah : line status.

al : receive character.

4. get port status

ah register

always set to 3h.

dx register

port number.

return value (ax register)

line status and modem status.

5. activate port

ah register

always set to 4h.

dx register

port number.

return value

ax : 1954h if successful.

bl : maximum function number supported.

bh : rev of FOSSIL specification.

6. set DTR

ah register

always set to 6h.

al register

| 0 | turn off DTR. |
| --- | --- |
| 1 | turn on DTR. |

dx register

port number.

7. nondestructive read ahead

ah register

always set to Ch.

dx register

port number.

return value

ah : 0, if character is available.

al : the read character.

ax : FFFF, if no character is available.

The  character read by this operation is  not removed from input queue. This means that the same character will be obtained in next  read operation.

8. set flow control

ah register

always set to Fh.

al register

enable or disable flow control. When set to 1, means enable.

| bit 0 | enable / disable receiving XON/XOFF |
| --- | --- |
| bit 1 | enable / disable RTS/CTS. |
| bit 2 | enable /disable DTR/DSR. |
| bit 3 | enable / disable transmitting XON/XOFF |

dx register

port number.

9. receive block from a port

ah register

always set to 18h.

dx register

port number.

cx register

length of block.

es:di register

starting address of block.

return value (ax register)

count of received bytes.

10. send block to a port

ah register

always set to 19h.

dx register

port number.

cx register

number of bytes to be sent.

es:di register

starting address for received block.

return value (cx register)

count of sent bytes.

11. send break signal

ah register

always set to 1Ah.

dx register

port number.

al register

| 1 | send break signal. |
| --- | --- |
| 0 | stop to send break signal. |

12. return information about PCCOM driver

ah register

always set to 1Bh.

dx register

port number.

cx register

length of block.

es:di register

starting address of block.

This  function will transfer  information   about  PCCOM  and its   current status  to the  caller.

return value (ax register)

The  data structure  (assembly  language style)currently  returned by PCCOM  is  as follows:

;DEFINE BEGIN OF STRUCTURE

INFO      EQU  $

;SIZE OF THIS STRUC IN BYTES

STRSIZ    DW   INFO_SIZE

;FOSSIL SPECIFICATION REV

MAJVER    DB   CURR_FOSSIL

;REV OF THIS DRIVER.

MINVER    DB   CURR_REV

;"FAR" POINTER TO ASCIIZ

;DRIVER DESCRIPTION STRING.

IDENT     DD   ID_STRING

;BYTE SIZE OF THE INPUT BUFFER

IBUFR     DW   IBSIZE

;NUMBER OF BUFFERED RECV BYTES

IFREE     DW   ?

;BYTE SIZE OF THE XMIT BUFFER

OBUFR     DW   OBSIZE

;NUMBER OF BUFFERED XMIT BYTES

OFREE     DW   ?

;WIDTH OF DISPLAY SCREEN

SWIDTH    DB   SCREEN_WIDTH

;HEIGHT OF DISPLAY SCREEN

SHEIGHT   DB   SCREEN_HEIGHT

;BAUD RATE, COMPUTER TO MODEM

BAUD      DB   ?

13. get buffer status

ah register

always set to 30h.

dx register

port number.

return value (cx)

number of characters in buffer.

FFFh : overflow.

14. set port address

ah register

always set to 31h.

dx register

port number.

bx register

port address.

15. set vector address

ah register

always set to 32h.

dx register

port number.

bx register

vector address.

16. set active status in vector

ah register

always set to 33h.

dx register

port number.

al register

| 0 | Active low |
| --- | --- |
| 1 | Active high |

17. set modem control

ah register

always set to 34h.

dx register

port number.

al register

modem control value.

| bit 7 | 0 |
| --- | --- |
| bit 6 | 0 |
| bit 5 | 0 |
| bit 4 | Loopback |
| bit 3 | OUT2 |
| bit 2 | OUT1 |
| bit 1 | RTS |
| bit 0 | DTR |

18. get port parameters

ah register

always set to 35h.

dx register

port number.

return value(al)

communication parameters. (see 00H)

19. get modem control value

ah register

always set to 36h.

dx register

port number.

al register

modem control value.

20. get flow control value

ah register

always set to 37h.

dx register

port number.

return value (al)

flow control value. see 0fH.

21. get the total number of PCCOM sets

ah register

always set to 38h.

return value (ax)

total number of PCCOM sets.

22. get information about each PCCOM set

ah register

always set to 39h.

dl register

set number.

es:di register

starting address of block.

return value (ax)

number  of  bytes  returned  in   buffer point by es:di

register.

Data structure :

irq             db  ;interrupt request

portno      dw  ;total number of ports in

;this set

groupno   dw  ;total number of groups in

;this set

whichgroup  dw  ;specified port in which group

base         dw  ;base address for this com set

vector      dw  ;interrupt vector address

active       db  ;active hi or lo in vector

portno     db  ;total number of ports in

;this com set

:          :             :

:          :             :

base        dw  ;base address for this com set

vector     dw  ;interrupt vector address

active      db  ;active hi or lo in vector

portno    db  ;total number of ports in

Represented in C:

typedef struct {

unsigned char irq;

unsigned int  portno;

unsigned int  groupno;

unsigned int  whichgroup

struct {

unsigned int  base;

unsigned int  vector;

unsigned char active;

unsigned char portno;

} group[8];

} BiosComSet_t;

23. get the total number of PCCOM ports

ah register

always set to 3Ah.

return value (ax)

total number of ports.

24. detect whether the the port is existent

ah register

always set to 3Bh.

dx register

port number.

return value (ax)

| 55AA | this port exists. |  |
| --- | --- | --- |
| 0 |  | this port not existent. |

25. get information of one PCCOM port

ah register

always set to 3Ch.

dx register

port number.

es:di register

pointer   to   space   containing   port information.

turn value (ax)

Data structure :

addr            dw  ;port address

baud_rate  dw  ;baud rate

lcr               db  ;line control register value

mcr             db  ;modem control register value

fc                 db  ;flow control protocol

q_sz            dw  ;rx ring buffer size

q_free        dw  ;free space in ring buffer

rx_enable  db  ;flag to notify the port

;receivable

tx_enable  db  ;flag to notify the port

;transmission

Represented in C:

typedef struct {

unsigned int  addr;

unsigned int  baud;

unsigned char lcr;

unsigned char mcr;

unsigned char fc;

unsigned char rx_qsz;

unsigned int  rx_qfree;

unsigned char rx_enable;

unsigned char tx_enable;

} BiosComPort_t;

26. notify peer to stop sending data

ah register

always set to 3Dh.

dx register

port number.

27. notify peer to restart sending data

ah register

always set to 3Eh.

dx register

port number.

28. set time out value

ah register

always set to 3Fh.

dx register

port number.

cx register

time out value in unit of 1/18 sec.

29. get time out value

ah register

always set to 3Fh.

dx register

port number.

return value (cx)

time out value in unit of 1/18 sec.

6.5 PCCOM DOS IOCTL functions

Under  the  DOS interrupt  21H  and  its sub function 44H, user can use IOCTL to access PCCOM character device.

1. IOCTL input function

Input:

AH    = 44H

AL    = 02H

BX    = handle number

CX    = number of bytes to transfer

DS:DX = segment: offset of data buffer

(viewed as control block)

Output:

If function is successful:

Carry flag is clear.

AX    = number of bytes to transfer.

Data buffer at DS:DX contains data

read from device driver.

If function is not successful:

Carry flag is set.

AX = error code:

| 01H | invalid function |
| --- | --- |
| 05H | access denied |
| 06H | invalid handle |
| 0DH | invalid data (bad control block) |

The transfer address(DS:DX) points to  a control  block  that is used  to  communicate with the device driver. The first byte of the control block determines the request that  is being made.

| Code | Bytes | Function |
| --- | --- | --- |
| 0 | 3 | get serial port status |
| 1 | 3 | get port communication parameters |
| 2 | 3 | get modem control value |
| 3 | 3 | get flow control value |
| 4 | 3 | get the size of available data in rx buffer |

- Get Serial Port Status

PortSts  db  0     ;control block code

dw  ?     ;serial port status

The  format  of  serial  port  status  is referred to BIOS function 03H.

- Get Port Setting

PortParam  db  1   ;control block code

db  ?   ;port communication parameters

db  ?   ;not used

The   format  of  serial   port   setting parameters is referred  to BIOS function  00H.

- Get Modem Control Value

PortMC    db  2    ;control block code

db  ?    ;modem control value

db  ?    ;not used

The  format  of modem  control  value  is referred to BIOS function 36H.

- Get Flow Control Value

PortFC    db  3    ;control block code

db  ?    ;flow control value

db  ?    ;not used

The  format  of  flow  control  value  is referred to BIOS

function 0FH.

- Get the Size of Available data in RX Buffer

PortRXData  db  4  ;control block code

dw  ?  ;available data size in rx buffer

2. IOCTL Output functions

Input:

AH    = 44H

AL    = 03H

BX    = handle number

CX    = number of bytes to transfer

DS:DX = segment:offset of data buffer

(viewed as control block)

Output:

If function is successful:

Carry flag is clear.

AX = number of bytes to transfer.

If function is not successful:

Carry flag is set.

AX = error code:

| 01H | invalid function |
| --- | --- |
| 05H | access denied |
| 06H | invalid handle |
| 0DH | invalid data (bad control block) |

The   transfer address(DS:DX) points to  a control  block  that is used  to  communicate with the device driver. The first byte of the control block determines the request that  is being made.

| Code | Bytes | Function |
| --- | --- | --- |
| 0 | 2 | set port communication parameters |
| 1 | 2 | set modem control value |
| 2 | 2 | set flow control value |
| 3 | 2 | turn on/off "break" |
| 4 | 2 | purge rx buffer |

- Set Port Communication Parameters

PortParam  db  0    ;control block code

db  XX  ;port communication parameters

This   function  is  the  same  as   BIOS function 00H.

- Set Modem Control Value

PortMC  db  1      ;control block code

db  XX     ;modem control value

This   function  is  the  same  as   BIOS function 34H.

- Set Flow Control Value

PortFC  db   2     ;control block code

db   XX    ;flow control value

This   function  is  the  same  as   BIOS function 0FH.

- Turn on/off "BREAK"

PortFC  db   2     ;control block code

db   XX  ; 1 : start sending 'break'

; 0 : stop sending 'break'

This   function  is  the  same  as   BIOS function 1AH.

- Purge in RX Buffer

PortRXData  db  4  ;control block code

db  ?  ;not used

This   function  is  the  same  as   BIOS function 0AH.

CHAPTER 7

DIAGNOSTIC PROGRAM

In this chapter we will introduce how to use  diagnostic  programs to prove the  PCCOM device driver is active and your serial  port is work normally.

7.1 TESTCOM Diagnostic Program

The TESTCOM program provides  diagnostic routine  to  test  your  serial  ports  under MS/DOS operating system. It provides internal loopback test and external loop test.

To  test your serial port under  MS/DOS, please type

A>  TESTCOM

then screen will display as follows:

wait a moment, the screen will display as follows:

PC COM. TEST PROGRAM V2.0

DECISION-COMPUTER INTERNATIONAL CO., LTD.

SETUP RS232

INTERNAL LOOP TEST

EXTERNAL LOOP TEST

AUTO TEST

SAVE CONFIGURATION

EXIT PROGRAM

1. SETUP RS232

You may specify communication parameters such  as baud rate, data bits ... etc.  Since these  communication  parameters are  set  up when computer system is boot. However, if you want  reset communication parameters,  please select  setup  RS232 function  to  setup  new communication parameters. We provide  several preconfigurated  file  of  our   multi-serial adapters,  which  are named  *.CFG,  you  may rename one of these files to TESTCOM.CFG.

2. INTERNAL LOOP TEST

The  internal  loop  test  provides   a loopback  feature for diagnostic  testing  of the  8250/16450/16550  chip.  You  may   type characters  from keyboard, then the  received character  is  display on  the  screen.  When something error such as time out, frame error ... etc., the reversed character will display on the screen to indicate communication fail. We note that some 16450 chip will not  emerge an   interrupt  when  loopback   feature   is selected.

3. EXTERNAL LOOP TEST

When you select external loop test, your typing  characters  will  be  sent  from  one serial  port to another serial  port.  Before you select external loop test, please connect pin  2 of send port to pin 3 of receiver port and  pin 3 of send port to pin 2 of  receiver port.  Since the serial communication is full duplex,  you  can loopback send and  received port in one port.

a. Connects two rs232 ports

pin 2  TxD            RxD   pin 3

pin 3  RxD            TxD   pin 2

b. Loopback

pin 2  TxD

pin 3  RxD

4. AUTO TEST

When  you select auto test,  the TESTCOM will  test  your serial  port  automatically. Both  internal  loop test and  external  loop test are used to check the selected port. The sending  character are read  from  README.DOC file.

5. SAVE CONFIGURATION

Save  communication  parameters to  disk file.  When TESTCOM is loaded,  it will  load the  preconfiguration  from  disk  file.  The configuration file name is TESTCOM.CFG.

You can use <RETURN> key to let  TESTCOM accepts  your selection during you  execution the  TESTCOM program,  and press <ESC> key to return the last menu.

APPENDIX A

VIRTUAL TERMINAL

VT.EXE  is a terminal evaluation program which is used to let your PC to active like a VT52  terminal.  Either the virtual  terminal features   and  file  transfer  features  are available in VT.EXE software,  moreover  some extended   graphics   capability   are   also supported.

A.1 Valid Control Sequence of VT

VT.EXE  supports  part of VT52  control sequence which are list in the following.

1. Cursor up                                              code= ESC A

Move  the  active position  upward  one position.

2. Cursor down                                          code= ESC B

Move  the active position downward  one position.

3. Cursor right                                            code= ESC C

Move the active position to the right.

4. Cursor left                                              code= ESC D

Move the active position to the left.

5. Cursor to home                                       code= ESC H

Move the cursor to the home position.

6. Reverse line feed                                     code= ESC I

Move  the  active position  upward  one position   without   altering   the    column position.  If  the active position is at  the top margin, a scroll down is performed.

7. Erase to end of screen                             code= ESC J

Erase  all  characters from the  active position to the end of

screen.

8. Erase to end of line                                  code= ESC K

Erase  all characters from  the  active position to the end of the current line.

9. Direct cursor address  code= ESC Y lin col

Move  the cursor to the specified  line (lin) and column (col).

A.2 Extended Graphics Capability

VT.EXE    provides   extended   graphics capability  which  may accept a  sequence  of graphics  control code (which is  represented by  ASCII  code)  to  draw  a  line,  circle, ellipse ...  etc. on the display monitor. The graphics  functions  can be used  under  EGA, VGA,   hercules,  and  8514  modes  or  other display   mode  that  supported  by  turbo  C graphics tools. The control sequence of these graphics   primitives   are   list   in   the following.

1. Enter graphics mode

code= ESC P

When  VT.EXE  accepts ESC  P,  then  the terminal  evaluator enter to graphics mode to ready accept graphics operation commands. The resolution of graphics mode depend on your PC monitor and adapter type which is detected by VT.EXE automatically.

2. Exit from graphics mode to text mode

code= Q

When  terminal evaluator is at  graphics mode, it can accept Q to enter text mode.

3. Move cursor to X,Y position

code= Mxxxxxyyyyy

Move  to (xxxxx,yyyyy)  position.  Where xxxxx represents  X coordinate,  which length is   five   digit.    yyyyy   represents    Y coordinate, which length is five digit.

4. Draw to X,Y position

code= Dxxxxxyyyyy

From    current   position    draw    to (xxxxx,yyyyy) position.

5. Draw a line

code= Lxxxxxyyyyywwwwwzzzzz

Draw   a  line  from  (xxxxx,yyyyy)   to (wwwww,zzzzz).

6. Draw a circle

code= Cxxxxxyyyyyrrrrr

Draw    a   circle   whose   center   is (xxxxx,yyyyy) and its radius is rrrrr.

7. Draw ellipse

code= Exxxxxyyyyyrrrrrsssss

Draw   a  ellipse  whose  center is   at (xxxxx,yyyyy),  and  the length of semi-major axis is rrrrr,  the length of semi-minor axis is sssss.

8. Draw a point

code= Pxxxxxyyyyy

Draw a point at (xxxxx,yyyyy).

9. Write a string

code= Sccccc<CR>

Draw  a graphics string on  the  screen, the string are quoted between S and <CR>. The maximum string length is 80 characters.

10. Select one color attribute

code= Axxxxx

Select  pen color,  the color number  is specified  by  xxxxx,  where xxxxx  has  five digit length.

We   will  give  some  example  to  show graphics ability in the section A.4.

A.3 How to Execute VT.EXE

After   PCCOM.SYS   is   executed   (For example,  set  up 3F8 port), just run  VT  to enter  virtual terminal evaluation mode.  The screen you will see is shown below.

F1: HELP   Copyright DECISION-COMPUTER Co., Ltd.

You  may key in character from  keyboard to  communicate with host computer or  select function  to setup communication  parameters. All available function keys are shown in  the following.

F1   HELP

When press F1 key,  system will display functions that you can select.

F2   S/CTRL

Press  F2  key will send XOFF  code  to host computer to notice that virtual terminal is  too busy.  When you want set XON to  host computer again, please press Q/CTRL.

F3   SETUP

Setup the communication protocol such as baud rate, data bits, stop bits ... etc.

F4   C/CTRL

Press  F4  key  will send C/CTRL code to remote   computer  to  kill   the   executing process.

F5   LISTCFG

List the communication parameters on the screen.

F6   SAVECFG

Save  the setup communication parameters to disk file named VT.CFG.

F7   Download (receive data from host)

Receive  data from the remote site  then save these data to a user specified file.

F8   Upload (transmit data to remote site)

Transfer  a  user  specified  file  that located at PC (which is now run VT) to remote site.

F9   EXIT

Exit virtual terminal evaluator.

F10  SHELL

Fork  MS/DOS shell command  interpreter, the  terminal evaluator is still resident  on the   main  memory.   When  you  enter  shell command,  you may run any MS/DOS commands and programs, because you stay at MS/DOS mode. To enter VT again, please type EXIT.

A.3.1 Download Data from Host

When  you select F7,  system will ask  a file  name  which is used to save  data  that sent from remote computer.

Suppose   your   virtual   terminal   is connected  to XENIX operating system and  you want download a file aa from host to PC,  and

save it in PC.DOC file, the procedure is

cat aa

F7

PC.DOC

<CR>

Don’t enter <CR>

Select F7

Enter file name

A.3.2 Upload Data to Host

When  you select F8,  system will ask  a file name and send the file contents to  host computer.   Under   XENIX  operating  system, suppose  you want send a file PC.DOC from  PC to host computer and save it in aa file,  the upload procedure is

cat > aa

F8

PC.DOC

<CR>

A.3.3 Setup

When you select F3,  you may modify  the communication parameters. After communication parameters are setup,  you may use F6 to save the   configuration   and  F5  to  list   the configuration.  When  VT is loaded,  it  will check   VT.CFG   to   load  the   last   save configuration       automatically.        The communication  parameters  are  list  in  the following.

1. port number

Select which I/O port.

2. baud rate

Select communication speed.

3. data bits

Select data bits

4. stop bits

Select stop bits.

5. parity

Press SPACE bar to select parity.

6. ECHO

Press  SPACE  bar to select  local  ECHO ON/OFF.   When   your   host   computer   has ability   to   send   back    the    received character,  you must select local ECHO OFF to let   host   computer  prompt   your   typing character.

7. new line

Press  SPACE  bar  to  select  new  line ON/OFF. VT will add <CR> code when it receive <LF> at new line ON mode.

8. delay time

Select  delay time interval between  two transmitting   characters  when   upload   is processing.

A.4 Examples

The following control string will draw a figure which is shown in the next page. Don't forget to press any key to enter text mode.

<ESC>P

L00000000000010000000

L00100000000010000100

L00100001000000000100

L00000001000000000000

P0005000030

P0005200032

P0005400034

P0005600036

P0005800038

P0006000040

P0006200042

P0006400044

P0006600046

P0006800048

C001000010000050

C001000010000040

C001000010000030

C001000010000020

C001000010000010

C001000010000005

C001000010000015

C001000010000025

C001000010000035

C001000010000045

C001000010000055

M0010000100

D0030000200

D0009000090

Q

Where  <ESC> is an ASCII code of  <ESC>. We have not store <ESC>P to the VT.DOC  file, so  that  you must send it from  keyboard  to enter graphics mode.

F1: HELP   Copyright DECISION-COMPUTER Co., Ltd.

APPENDIX B

COMMUNICATION PARAMETERS

B.1 Communication parameters

baud rate

The  transmit  speed between two  serial ports.

parity

The   parity  bit  is  used   in   error detection. Three different settings are used, which are ODD, EVEN, and NONE parity.

data bits

The  number of bits in each  transmitted or  received serial character.  Either 7 or 8 data bits can be specified.

stop bits

The  stop bits is used to  detect  where one character ends and another starts. Either 1  or 2 stop bits can be specified. When stop bits   is   specified  then  stop  bits   are generated  or  checked  in  the  transmit  or

receive data respectively. IRQ specifies   which  interrupt is  emerged when  the character is received  from  serial port.

interrupt vector

The   interrupt   vector  is  a   global interrupt  indicator  which  is  designed  at several  multi-serial adapters (such as  four serial  card, eight serial card ... etc.)  to check  which serial port  emerges  interrupt. Since  several serial ports are  designed  at one  adapter and use the common IRQ, so  that interrupt  vector is useful to  detect  which port emerges interrupt. For example,  suppose interrupt vector address of hardware  adapter is set to 2C2H, the relationship between each port  corresponds to the interrupt  indicator is shown in the follows. If you specify  this interrupt  vector address, when interrupt  is emerged, device driver may check address 2C2H to find which port emerges interrupt.

| bit 0 | port 1 |
| --- | --- |
| bit 1 | port 2 |
| bit 2 | port 3 |
| bit 3 | port 4 |
| bit 4 | port 5 |
| bit 5 | port 6 |
| bit 6 | port 7 |
| bit 7 | port 8 |

The  standard  COM1 and  COM2  need  not interrupt vector. We will list several famous multi-serial adapters in the appendix D.

modem control register

The   contents  of  the  modem   control register are indicated and described below.

| bit 0 | DTR |
| --- | --- |
| bit 1 | RTS |
| bit 2 | OUT1 |
| bit 3 | OUT2 |
| bit 4 | LOOP(0) |
| bit 5 | 0 |
| bit 6 | 0 |
| bit 7 | 0 |

The  bit  0  (DTR) and bit 1  (RTS)  are hardware  handshaking.  To let PCCOM  in  the normal mode, you must set the bit 0 and bit 1 to  1.  The bit 4 provides a loopback feature for diagnostic testing of the serial port. In this case,  you must set bit 4 to be 0.  OUT1 and  OUT2 are auxiliary user designed  output which is used to enable/disable interrupt  in most adapters,  so that you must control OUT1 and  OUT2  carefully to let your serial  port emerges an interrupt.  You may set OUT1=0 and OUT2=1  to  enable  interrupt  of  the   most standard COM1 and COM2 adapter,  however some adapters  may not work in this case,  you can ask   OUT1  and  OUT2  control  from   origin hardware manufacturer or trial and  error  to find one of four  combination  (0,0),  (0,1), (1,0),  (1,1).  Appendix D will list OUT1 and OUT2 signal of our multi-serial family.  Some 8250/16450  chip will generate OUT1 =  1  and OUT2 = 1 when internal loop is selected  (bit 4 is set to 1), so that they will not  emerge an  interrupt when internal loop  feature  is test.

active status

The  active  status is used  to  specify activation status of interrupt vector. Either active low or high can be set.  If active low is specified,  when the corresponding bit  of interrupt    vector   is   0,    means    the corresponding  port  emerges  an   interrupt, otherwise if the bit is 1, means no interrupt

is active.

XON/XOFF

XON/XOFF is software handshaking,  which is  used  to prevent the lost of  data  while communication   between  two  serial   ports. Suppose  we connect two computer named  local computer   and  remote  computer,   if  local computer  is  too busy to receive  data  from remote computer,  it send XOFF (S/CTRL  whose code  is 17) to remote computer to notice  he is   in  busy  mode.   When  remote  computer receives  XOFF,  it  will  stop  transmission until  the local computer sends  XON  (Q/CTRL whose  code is 19) to continue  transmission. The  same  manner  is  activated  at   remote computer.

DTR and DSR

The  DTR/DSR  is  hardware  handshaking. Data Set Ready(DSR) indicates that the modem or   data   set   is   ready   to   establish communications  link and transfer  data  with the    serial   interface.    Data   Terminal Ready(DTR) informs the modem or data set that the    serial   interface   is    ready    to communication.

RTS and CTS

The  RTS/CTS  is  hardware  handshaking. Clear  To Send(CTS) and Request To  Send(RTS) can   be  used  to  establish   communication protocol.  RTS  informs the modem or data set that   the  serial  interface  is  ready   to transmit data.

B.2 Serial Ports status

The serial ports status are described in the following.

Carrier Detected

The  modem  status which  provides  Data Carrier Detected(DCD) to indicate the voltage level of the associate RS232 line.

Ring Indicator(RI)

Indicates  that a telephone ring  signal has been received by the modem or data set.

Txmtr SR

Transmitter shift register of 8250/16450 chip.

Txmtr HR

Transmitter    holding    register    of  8250/16450 chip.

Break Received

Indicates   whether  the  received  data input is held in the spacing state for longer than a full word transmission time,  that is, the  total  time of start bit + data  bits  + parity bit + stop bits.

Framing Error

Indicates  that the  received  character did not have a

valid stop bit.

Parity Error

Indicates   that   the   received   data character  does not have the correct even  or odd parity.

Overrun Error

Indicates  that  data  in  the  receiver buffer   register   was  not  ready  by   the processor  before  the  next  characters  was transferred   into   the   receiver    buffer register,  thereby  destroying  the  previous character.

Receiver DR full

Received Data Register full. If the flag always  set, it means the RS232 port hang  up because  no  interrupt  generated  when  data arrive.  Please  restart your  computer  when error occurs.

APPENDIX C

SYNTAX SPECIFICATION OF HARDWARE CONFIGURATION

C.1 Syntax specification

Config := /B:Bios_no /D:Dev_name /A:Pccom

Bios_no := dec_value

Dev_name := Dev_strDev_no, Dev_name

:= Dev_name

[note : length(Dev_name) <= 8]

Dev_str := ([a..z] | [A..Z]))*

Dev_no := 1..99

Pccom := Com_irq, Pccom

:= Com_irq

:= nil

Com_irq := [Irq:Com_set]

Com_set := Com_vect:Com_set

:= Com_vect

:= nil

Com_vect := Base,Portno,Vector,Active:Com_port

Com_port := Port, Com_port

:= Port

:= nil

Port := (Buf_siz:Baud_rate:Line_control

:Modem_control:Flow_control)

:= (Buf_siz:Baud_rate:Line_control

:Modem_control:Flow_control)

* dec_value

Irq := 2 | 3 | 4 | 5 | 6 | 7 | 10 | 11 | 12

| 14 | 15

Base := hex_value

Portno := 1 | 2 | 4 | 8

Vector := hex_value | 0

Active := HI | LO

Buf_siz := dec_value k

:= dec_value

:= nil

Baud_rate := dec_value

Line_control := Parity-Data-Stop

Parity := E | O | N

Data :=  5 | 6 | 7 | 8

Stop := 1 | 2

Modem_control := Mc+Modem_control

:= Mc

:= nil

Mc = DTR | RTS | OUT1 | OUT2

Flow_control := Fc+Flow_control

:= Fc

:= nil

Fc = DTR | RTS | XON

Comment := # string

[a whole line comment]

dec_value := decimal number

hex_value := hexadecimal number

C.2 Some Constrains on Configuration Rule

%  The  size of total buffer in  one  Com_set    must  be less than 64K. However, the  size    of  total  buffer used in  driver  can  be    larger than 64K.

%  The  number of ports in one  Com_set  must    matches the  number of  Com_ports  in  the    following definition.

%  The maximum size of the  buffer  allocated    for one port is 32K, but The maximum  size    of  the  buffer allocated for one  set  is  64K.

%  In Com_port definition, buffer size,  baud    rate, and line control must be  specified, both modem control and flow control  could    be not specified if you do not want  these  capabilities,  however the  separator  ":"    can not be skipped.

C.3 How to install and configure PCCOM driver

%  The PCCOM driver that is DOS-style  driver    is nstalled when system boots by means of  inserting the following line in config.sys

DEVICE=C:PCCOM.SYS @c:\pccom.opt

The  content of the file  "pccom.opt"  is configuration    rule   specified;    the  character  '@' must be prefixed with  the configuration file.

APPENDIX D

MULTI-SERIAL PRODUCTS

PCCOM  software  is  a  powerful  device driver,  which  can be link to  any  hardware that provides serial port. In this  appendix, we will introduce how to setup PCCOM software when  use  our  products. We  note  that  all examples  are  default setting  that  satisfy origin  factory setting, however you may  set different hardware configuration if you need.

D.1 PCCOM 4 port card

PCCOM ISA bus 4 port card

SafLine 4 port card

Four serial adapter provides four serial ports, the origin factory setting are shown in the following.

| port 1 address | 2A0 |
| --- | --- |
| port 2 address | 2A8 |
| port 3 address | 2B0 |
| port 4 address | 2B8 |
| interrupt vector address | 2BF |
| interrupt active status | low |
| interrupt | IRQ5 |
| modem control register | RTS+DTR |

To   setup  four  serial  adapter,   the commands are shown in the PCCOM4.OPT file.

/B:2

/D:COM3

/A:[5:

2A0,4,2BF,LO:

(2k:9600:N-8-1:RTS+DTR:XON) * 4]

D.2 Eight Serial Adapter

Eight  serial  adapter  provides   eight serial ports,  the origin factory setting are shown in the following.

| port 1 address | 2A0 |
| --- | --- |
| port 2 address | 2A8 |
| port 3 address | 2B0 |
| port 4 address | 2B8 |
| interrupt vector address | 2BF |
| interrupt active status | low |
| interrupt | IRQ5 |
| port 5 address | 1A0 |
| port 6 address | 1A8 |
| port 7 address | 1B0 |
| port 8 address | 1B8 |
| interrupt vector address | 1BF |
| interrupt active status | low |
| interrupt | IRQ7 |
| modem control register | RTS+DTR |

To   setup  eight serial  adapter,   the commands are shown in the 8PORT.OPT file.

/B:2

/D:COM3

/A:[5:

2A0,4,2BF,LO:

(2k:9600:N-8-1:RTS+DTR:XON) * 4],

[7:

1A0,4,1BF,LO:

(2k:9600:N-8-1:RTS+DTR:XON) * 4]

D.3 Dual Eight Serial Adapter

Dual eight serial adapter provides eight serial ports,  the origin factory setting are shown in the following.

| port 1 address | 290 |
| --- | --- |
| port 2 address | 298 |
| port 3 address | 2A0 |
| port 4 address | 2A8 |
| port 5 address | 2B0 |
| port 6 address | 2B8 |
| port 7 address | 2C0 |
| port 8 address | 2C8 |
| Interrupt vector address | 2D0 |
| interrupt active status | high |
| interrupt | IRQ4 |
| modem control register | RTS+DTR |

To setup dual eight serial adapter,  the commands are shown in the DUAL8.OPT file.

/B:2

/D:COM3

/A:[4:

290,8,2D0,HI:

(2k:9600:N-8-1:RTS+DTR:XON) * 8]

D.4 PCCOM 8 port card

PCCOM ISA bus 8 port card

SafLine 8 port card

Super   Eight  serial  adapter  provides eight  serial  ports,   the  origin   factory setting are shown in the following.

| port 1 address | 280 |
| --- | --- |
| port 2 address | 288 |
| port 3 address | 290 |
| port 4 address | 298 |
| port 5 address | 2A0 |
| port 6 address | 2A8 |
| port 7 address | 2B0 |
| port 8 address | 2B8 |
| interrupt vector address | 2C2 |
| interrupt active status | low |
| interrupt | IRQ4 |
| modem control register | RTS+DTR |

To setup super eight serial adapter, the commands are shown in the PCCOM8.OPT file.

/B:2

/D:COM3

/A:[4:

280,8,2C2,LO:

(2k:9600:N-8-1:RTS+DTR:XON) * 8]

D.5 Standard COM1 and COM2

The  interrupt setting and port  address of  standard  COM1 and COM2 are shown in  the following.

| COM1 address | 3F8 |
| --- | --- |
| COM2 address | 2F8 |
| interrupt for COM1 | IRQ4 |
| interrupt for COM2 | IRQ3 |
| modem control register | RTS+DTR+OUT2 |

To setup COM1 and COM2, the commands are shown in the COM1COM2.OPT file.

/B:0

/D:COM1

/A:[4:

3F8,1,0,LO:

(8k:9600:N-8-1:RTS+DTR+OUT2:XON)],

[3:

2F8,1,0,LO:

(8k:9600:N-8-1:RTS+DTR+OUT2:XON)]

D.6 SafLine 16 port card

SafLine  16 port card is  composed  with two PCCOM 8 port cards, you can set it as two PCCOM    8   port   cards.    The    hardware configuration   file   is   shown   in    the PCCOM16.OPT file.

D.7	PCCOM 2 Port PCI card

This PnP PCI card products 2 serial ports, and can be set in the OPT file as follows:

/B:2

/D:COM3

/P: [:2: (1K: 9600: N-8-1 :: XON) *2]

D.8  PCCOM 4 Port PCI card

This PNP PCI card provides 8 serial ports and can be set on the OPT file as follows:

/B:2

/D: COM3

/P: [:4: (1K :P600: N-8-1 :: XON) *4]

D.9  PCCOM 8 Port PCI card

This PNP PCI card provides 8 serial ports and can be set on the OPT file as follows:

/B:2

/D: COM3

/P: [:8: (1K:9600 :N-8-1:XON )*8]

APPENDIX E

WARRANTY INFORMATION

E.1 Copyright

Copyright  1993,1994  DECISION  COMPUTER INTERNATIONAL CO., LTD. all rights  reserved. No  part of PCCOM software and manual may  be reproduced,   transmitted,  transcribed,   or translated  into  any  language  or  computer language,  in  any  form  or  by  any  means, electronic,  mechanical,  magnetic,  optical, chemical,  manual, or otherwise, without  the prior written permission of DECISION COMPUTER INTERNATIONAL  CO.,  LTD., 4F, No. 31,  Alley 4,  Lane  36,  Sec. 5,  Min-Shen  East  Road,

Taipei,  Taiwan, R.O.C. FAX :  886-2-7665702, TELEX : 16059 DECISION, TEL : (02)  766-5753, 7659782, 769-5786.

Each piece of PCCOM package permits user to  use  PCCOM only on a single  computer,  a registered  user  may use the  program  on  a different  computer,  but  may  not  use  the program on more than one computer at the same time.

Corporate licensing agreements   allow duplication and distribution of   specific number   of   copies within   the   licensed institution.  Duplication of multiple copies is not allowed except through execution of a licensing   agreement.   Welcome   call   for details.

G.2 Warranty Information

PCCOM warrants that for a period of one year from the date of purchase   (unless otherwise specified in the warranty card) that   the   goods supplied   will   perform according to the specifications defined in the user manual.  Furthermore that the  PCCOM  product will be supplied free from defects in

Materials   and workmanship and be   fully functional under normal usage.

In  the event of the failure of a  PCCOM product within the specified warranty period, PCCOM will, at its option, replace or  repair  the  item  at  no  additional  charge.   This limited   warranty  does  not  cover   damage resulting  from  incorrect  use,   electrical interference,  accident, or  modification  of the product.

All  goods returned for warranty  repair must  have the serial number  intact.   Goods without  serial numbers attached will not  be covered by the warranty.

Transportation  costs for goods returned must  be  paid by  the  purchaser.   Repaired goods  will be dispatched at the  expense  of PCCOM.

To  ensure  that your PCCOM  product  is covered  by  the warranty provisions,  it  is necessary that you return the Warranty card.

Under  this  Limited  Warranty,  PCCOM's obligations  will  be limited  to  repair  or replacement  only,  of  goods  found  to   be defective  as  specified  above  during   the warranty period.  PCCOM is not liable to  the purchaser  for any damages or losses  of  any kind,  through  the use of, or  inability  to use, the PCCOM product.

PCCOM  reserves the right  to  determine what    constitutes   warranty   repair    or replacement.

Return  Authorization:  It is necessary  that any returned goods are clearly marked with an RA  number  that has been  issued  by  PCCOM.  Goods  returned  without  this  authorization

will not be attended to.